K0WLY Two-Way CW Keyer

LilyGO T-Display S3 AMOLED

Operator's Manual — Preliminary

Version 1.6.0

K0WLY — Saratoga Springs, Utah — Grid DN40

License

Copyright © 2026 K0WLY (Carl Cowley) — CERN-OHL-W v2 / CC BY 4.0

73 de K0WLY

# 1. Overview

The K0WLY Two-Way CW Keyer is an ESP32-S3 based iambic keyer on the LilyGO T-Display S3 AMOLED (v1.6.0).

- Iambic Mode A, Iambic Mode B or Ultimatic keyer, 5–40 WPM, straight key mode

- Farnsworth spacing — character speed plus overall (effective) speed, e.g. 20/10

- Key output via PC817 optocoupler (GPIO12)

- Iambic DIT/DAH output (GPIO40/41) for radios with built-in keyers

- Paddle reverse and keyer mode (A/B/Ultimatic) via GPIO16

- Adjustable sidetone 400–900 Hz, logarithmic volume

- Head copy delay 0–3s or Audio Only mode

- Word gap spacing (off or 4–9 dits)

- WiFi AP (K0WLY-XXXX) with web interface for settings and file management

- Text file playback with pause/resume, drives iambic output

- Automatic ESP-NOW peer-to-peer pairing, two-way CW

- Full Morse decoding: A–Z, 0–9, punctuation, prosigns

# 2. Display Layout

| Zone | Location | Content |
| --- | --- | --- |
| Header Bar | Top 32px | WPM, Hz, DLY, VOL, GAP, A/B/U/SK, SOLO/DUAL |
| TX Line | Upper middle | Outgoing chars (green) |
| RX Line | Lower middle | Incoming chars (cyan) |
| Status Area | Bottom 96px | Pot mode, value, peer MAC, unit ID, version |

## 2.1 Header Bar

- WPM or WPM/FARN — character speed / overall (effective) speed

- Hz — sidetone frequency

- DLY — head copy delay or AO

- VOL — volume percentage

- GAP — word gap setting

- A / B / U / SK — Iambic Mode A, Iambic Mode B, Ultimatic, or Straight Key

- SOLO / DUAL — ESP-NOW peer status

# 3. Controls and Connections

## 3.1 GPIO Pin Assignments

| Function | GPIO | Notes |
| --- | --- | --- |
| DIT Paddle Input | GPIO11 | Active LOW, ext 10kΩ pull-up |
| DAH Paddle Input | GPIO10 | Active LOW, ext 10kΩ pull-up |
| Key Out | GPIO12 | HIGH=keyed, PC817 optocoupler |
| Sidetone | GPIO13 | PWM → 470Ω → 2N4401 → speaker/headphones |
| Pot (ADC) | GPIO14 | Wiper of 10–20kΩ pot |
| Key Mode | GPIO15 | SPST: open=iambic, closed=straight key |
| Paddle Reverse / Keyer Mode | GPIO16 | Short=dit/dah swap, Long=cycle Mode A/B/U |
| Pot Mode Select | GPIO39 | PBNO: short=cycle, long=edit |
| Iambic DIT Out | GPIO40 | Active HIGH, PC817 optocoupler to radio DIT |
| Iambic DAH Out | GPIO41 | Active HIGH, PC817 optocoupler to radio DAH |

## 3.2 GPIO16 Button Functions

| Press | Action |
| --- | --- |
| Short press | Toggles dit/dah swap (also available on web page) |
| Long press (1 sec) | Cycles keyer mode A → B → U (Ultimatic) → A — header shows A, B or U |

## 3.3 Keyer Modes

Mode A (default) — when both paddles are released during an element, the keyer finishes that element and stops.

Mode B — when both paddles are released during an element, the keyer sends one additional opposite element before stopping. Preferred by some operators for smoother squeeze keying.

Ultimatic (U) — when both paddles are squeezed, the paddle pressed most recently wins and repeats instead of alternating. Release it and the keyer goes to the paddle still held, or stops if none is. A new tap of the opposite paddle during an element is remembered and sent next. A paddle that is simply held down is not remembered, so letting go of both paddles stops the keyer after the current element.

NOTE: Keyer mode (A/B/U) is saved to flash and restored on power cycle.

## 3.4 Key Output (GPIO12)

Drives a PC817 optocoupler to key the transceiver. Active HIGH. Never connect GPIO12 directly to a radio.

## 3.5 Iambic DIT/DAH Output (GPIO40/41)

For radios with built-in keyers. Active HIGH (GPIO HIGH = LED on = radio keyed). Both outputs driven during file playback. Paddle reverse applies to outputs.

- Tip = DIT (GPIO40 optocoupler)

- Ring = DAH (GPIO41 optocoupler)

- Sleeve = GND

NOTE: Set the radio's WPM to match the keyer's CHARACTER speed. Farnsworth gaps are transparent to the radio.

## 3.6 Audio Circuit

Speaker — direct, NO coupling capacitor:

3.3V → Speaker → Collector / 2N4401 / GPIO13 → 470Ω → Base / Emitter → GND

Headphone — with coupling cap:

3.3V → 1kΩ → Collector / 2N4401 #2 / Emitter → GND. Collector → 10kΩ volume pot → 47µF bi-polar → 100Ω → 3.5mm tip.

# 4. Parameter Adjustment

## 4.1 Pot Mode Button

| Press | Action |
| --- | --- |
| Short press | Cycle: WPM → FREQ → DELAY → VOL → GAP → WPM |
| Long press (1s) | Enter/exit edit mode |
| Short press (in edit) | Exit edit and advance to the next parameter (in WPM, the first short press moves to the Farnsworth step). Values save as you adjust |

## 4.2 Parameter Ranges

| Parameter | Range | Notes |
| --- | --- | --- |
| Char Speed | 5–40 WPM | PARIS standard |
| Farnsworth | 4 WPM – char speed | Overall (effective) speed |
| Frequency | 400–900 Hz | Each unit independent |
| Head Copy Delay | 0.0–3.0s or AO | Top 10% = Audio Only |
| Volume | 2–100% | Logarithmic |
| Word Gap | OFF or 4–9 dits | Bottom 20% = OFF |

# 5. WiFi Operation

## 5.1 Access Point

Each unit creates K0WLY-XXXX (XXXX = last 4 hex of MAC). No password. Browse to http://192.168.4.1.

## 5.2 Web Interface

- Settings tiles — Char Speed, Farnsworth, Frequency, Volume, Head Copy Delay and Word Gap each have − and + buttons

- Keyer Mode tile — tap Iambic A, Iambic B or Ultimatic

- Paddles tile — tap Dit-Dah or Dah-Dit (same as the hardware paddle reverse button)

- Changes are saved on the keyer, and the page picks up changes made with the knob and buttons every 2 seconds

- Farnsworth shows OFF when it equals the character speed

- On the web page, character speed steps 38 ↔ 40 (39 is skipped, since 39 and 40 WPM share the same 30 ms dit). The knob and buttons can still select 39

- Practice Files — Play (▶) and Delete (✕) per file

- Stop Playback — immediate

- Pause/Resume — resumes from same position

- Upload Text File — .txt files only

## 5.3 Text File Playback

- TX line clears on start, characters appear as they sound

- A newline counts as one word gap; blank lines and a newline right after a space add nothing extra. Spaces you type are always kept

- KEY OUT and iambic outputs driven during playback

- Elements transmitted to peer over ESP-NOW

## 5.4 ESP-NOW Peer

Units auto-discover on channel 1. Header shows DUAL (green) when paired.

# 6. Firmware Information

| Item | Details |
| --- | --- |
| Platform | ESP32-S3 (LilyGO T-Display S3 AMOLED) |
| Framework | Arduino via PlatformIO |
| Board Definition | lilygo-t-amoled (custom boards/ folder) |
| WiFi | AP+STA: ESPAsyncWebServer + ESP-NOW |
| File Storage | LittleFS on internal flash |
| Keyer Task | FreeRTOS Core 1, 1ms resolution |
| Keyer Modes | Mode A, Mode B and Ultimatic (GPIO16 long press) |
| NVS Version | 4 |
| CW Standard | PARIS (ITU-R M.1677-1) |
| License | CERN-OHL-W v2 / CC BY 4.0 |

# 7. Default Settings

| Setting | Default |
| --- | --- |
| Speed | 20 WPM |
| Farnsworth | Off |
| Sidetone Frequency | 700 Hz |
| Head Copy Delay | 0.0 seconds |
| Volume | 64% |
| Word Gap | OFF |
| Keyer Mode | Mode A |
| Paddle Reverse | Normal |
| AP SSID | K0WLY-XXXX (unique per unit) |

# 8. Troubleshooting

| Symptom | Likely Cause | Fix |
| --- | --- | --- |
| Display blank on boot | Crash | Hold BOOT, press RST, reflash |
| Always SOLO | Peer out of range | Both units on channel 1 |
| Wrong characters | Paddles reversed | Short press GPIO16 or web swap |
| No sidetone | Volume at min | Enter VOL edit, increase |
| No speaker audio | Coupling cap installed | Remove cap |
| Radio not keying | Optocoupler issue | Check GPIO12 circuit |
| G90 wrong speed | G90 WPM mismatch | Set G90 WPM = char speed |
| K0WLY-XXXX not visible | AP not started | Check serial monitor |
| Files not showing | LittleFS issue | Reflash with correct board |

# Morse Code Reference

Letters:

| A ·− | B −··· | C −·−· | D −·· | E · | F ··−· |
| --- | --- | --- | --- | --- | --- |
| G −−· | H ···· | I ·· | J ·−−− | K −·− | L ·−·· |
| M −− | N −· | O −−− | P ·−−· | Q −−·− | R ·−· |
| S ··· | T − | U ··− | V ···− | W ·−− | X −··− |
| Y −·−− | Z −−·· |  |  |  |  |

Numbers:

| 0 −−−−− | 1 ·−−−− | 2 ··−−− | 3 ···−− | 4 ····− | 5 ····· |
| --- | --- | --- | --- | --- | --- |
| 6 −···· | 7 −−··· | 8 −−−·· | 9 −−−−· |  |  |

Punctuation:

| . period ·−·−·− | , comma −−··−− | ? question ··−−·· | / slash −··−· | ( open −·−−· | ) close −·−−·− |
| --- | --- | --- | --- | --- | --- |
| : colon −−−··· | ; semi −·−·−· | - hyphen −····− | ' apos ·−−−−· | " quote ·−··−· | @ at ·−−·−· |
| & amp ·−··· | ! excl −·−·−− | _ under ··−−·− | $ dollar ···−··− |  |  |

Prosigns:

| + AR ·−·−· end msg | = BT −···− break | ~ SK ···−·− end QSO | & AS ·−··· wait |  |  |
| --- | --- | --- | --- | --- | --- |

K0WLY Two-Way CW Keyer — Preliminary Manual v1.6.0 — 73 de K0WLY