K0WLY Two-Way CW Keyer

User Guide

Version 1.6.0

K0WLY — Saratoga Springs, Utah — Grid DN40

# 1. What Is the K0WLY CW Keyer?

The K0WLY Two-Way CW Keyer is a Morse code practice and operating tool. Connect your iambic paddle and get:

- Sidetone speaker — hear what you send

- Scrolling display showing characters you send and receive

- Two-way practice — connect two units wirelessly

- Text file playback — load practice files from your phone

- Radio keying output — connect to your transceiver

- Iambic Mode A, Iambic Mode B or Ultimatic — selectable via long press

# 2. Understanding the Display

| Area | What It Shows |
| --- | --- |
| Header (top) | Speed, frequency, delay, volume, gap, mode, peer status |
| TX line (green) | Characters you send or play from file |
| RX line (cyan) | Characters received from other unit |
| Status area (bottom) | Current setting, peer info, unit ID and version |

## 2.1 Header Bar

- WPM or WPM/FARN — sending speed

- Hz — sidetone pitch

- DLY — head copy delay (0.0s = off, AO = Audio Only)

- VOL — volume

- GAP — word gap (OFF or 4–9)

- A / B / U / SK — Iambic Mode A, Iambic Mode B, Ultimatic, or Straight Key

- SOLO / DUAL — whether a second unit is connected

# 3. Controls

## 3.1 Knob and Mode Button

| Action | Result |
| --- | --- |
| Short press | Cycle: WPM → FREQ → DELAY → VOL → GAP → WPM |
| Long press (1s) | Enter or exit edit mode — [EDIT] appears |
| Turn knob (in edit) | Adjust value |
| Short press (in edit) | Exit edit mode and move to the next setting. Values save as you turn the knob |

## 3.2 Paddle Reverse Button

| Press | Action |
| --- | --- |
| Short press | Toggle dit/dah swap |
| Long press (1s) | Cycle keyer mode A → B → U (Ultimatic) → A — header shows A, B or U |

## 3.3 Keyer Modes

Mode A — keyer stops after the current element when both paddles are released. Most common mode.

Mode B — keyer sends one extra opposite element after both paddles are released. Some operators prefer this for smoother squeeze keying.

Ultimatic (U) — when both paddles are squeezed, the last paddle pressed wins and repeats instead of alternating. Let go of it and the keyer goes to the paddle still held, or stops if none is. A quick tap of the other paddle during an element is remembered, but a paddle you are just holding down is not, so letting go of both stops the keyer.

TIP: Keyer mode (A/B/U) is saved and restored on power cycle.

## 3.4 Setting WPM and Farnsworth

- Long press WPM — set character speed

- Short press — set overall (effective) speed (4 WPM min, up to char speed). At 20/10, characters are sent at 20 WPM and the spacing is stretched so the overall speed is 10 WPM

- Short press — exit. Header shows 25WPM or 25/10

TIP: When using the iambic output to a radio with a built-in keyer, always set the radio's WPM to match your CHARACTER speed (first number). Farnsworth gaps are transparent to the radio and work correctly at any Farnsworth setting.

# 4. Using Your Phone

## 4.1 Connecting

- Connect to WiFi network K0WLY-XXXX (XXXX matches unit ID on display)

- No password required — tap Connect Anyway if warned about no internet

- Open browser — go to http://192.168.4.1

## 4.2 Web Page

Settings

- Settings tiles — Char Speed, Farnsworth, Frequency, Volume, Head Copy Delay and Word Gap each have − and + buttons

- Keyer Mode tile — tap Iambic A, Iambic B or Ultimatic

- Paddles tile — tap Dit-Dah or Dah-Dit (same as the hardware paddle reverse button)

- Changes are saved on the keyer, and the page picks up changes made with the knob and buttons every 2 seconds

- Farnsworth shows OFF when it equals the character speed

- On the web page, character speed steps 38 ↔ 40 (39 is skipped, since 39 and 40 WPM share the same 30 ms dit). The knob and buttons can still select 39

Practice Files

- Tap ▶ to play a file

- Tap ✕ to delete a file

- File names show without .txt, underscores as spaces

Stop / Pause / Resume

- Stop Playback — immediate stop, clears buffer

- Pause/Resume — instant pause, resumes from same position

Upload Text File

Select any .txt file from your phone and tap Upload.

## 4.3 Practice File Tips

- Use plain text files (.txt)

- Spaces between words become CW word gaps

- A new line counts as one word gap; blank lines add no extra pause. Spaces you type are always kept

- Name files descriptively: cq_de_k0wly.txt → shows as 'cq de k0wly'

- All punctuation and prosigns are supported

# 5. Two-Way CW Practice

Two units auto-discover each other and show DUAL in green. No setup needed.

- TX line (green) — what you send

- RX line (cyan) — what the other station sends

- Each unit plays at its own sidetone pitch

- Head copy delay applies to incoming display only — audio always immediate

## 5.1 Head Copy Delay

- 0.0s — normal, characters appear immediately

- 0.5s–3.0s — try to copy before looking

- AO (Audio Only) — characters never appear, pure audio copy

# 6. Connecting to a Transceiver

## 6.1 Key Output

Connects to your radio's straight key input. Keyer controls exact timing.

## 6.2 Iambic Output

For radios with built-in keyers (e.g. Xiegu G90). 3.5mm stereo jack:

- Tip — DIT

- Ring — DAH

- Sleeve — GND

NOTE: Set the radio's WPM to match the keyer's character speed. With Farnsworth (e.g. 25/10) set the radio to 25 WPM. This applies to all radios with built-in keyers.

# 7. Default Settings

| Setting | Default |
| --- | --- |
| Speed | 20 WPM |
| Farnsworth | Off |
| Sidetone Frequency | 700 Hz |
| Head Copy Delay | 0.0 seconds |
| Volume | 64% |
| Word Gap | OFF |
| Keyer Mode | Mode A |
| Paddle Reverse | Normal |

# 8. Updating Firmware

## 8.1 Windows — Espressif Flash Download Tool

- Download from espressif.com/en/support/download/other-tools

- Select ESP32-S3, browse to firmware.bin, set address 0x10000

- Select COM port, baud 921600, click START

- If fails: hold BOOT, press RST, release BOOT, click START

## 8.2 Mac/Linux — esptool.py

pip install esptool

esptool.py --port /dev/tty.usbmodem101 --baud 921600 write_flash 0x10000 firmware.bin

# 9. Practice Tips

- Start at comfortable speed — accuracy beats speed

- Use Farnsworth — char speed 20-25 WPM, effective 5-10 WPM

- Increase head copy delay gradually toward Audio Only

- Practice with real QSO text, call signs, contest exchanges

- Try Iambic Mode A, Mode B and Ultimatic — find which feels most natural

- Two-unit practice is the closest thing to a real QSO

# 10. Quick Reference

| Task | How |
| --- | --- |
| Change speed | Short press → WPM → long press → turn → short press |
| Set Farnsworth | Long press WPM → char speed → short press → overall speed → short press |
| Swap dit/dah | Short press paddle reverse button OR web page |
| Change keyer mode (A/B/U) | Long press paddle reverse button |
| Connect phone | WiFi → K0WLY-XXXX → browser → 192.168.4.1 |
| Play a file | Web page → ▶ next to file name |
| Pause playback | Web page → Pause/Resume |
| Stop playback | Web page → Stop Playback |
| Audio Only mode | Short press → DELAY → long press → turn to max → short press |

# Morse Code Reference

Letters:

| A ·− | B −··· | C −·−· | D −·· | E · | F ··−· |
| --- | --- | --- | --- | --- | --- |
| G −−· | H ···· | I ·· | J ·−−− | K −·− | L ·−·· |
| M −− | N −· | O −−− | P ·−−· | Q −−·− | R ·−· |
| S ··· | T − | U ··− | V ···− | W ·−− | X −··− |
| Y −·−− | Z −−·· |  |  |  |  |

Numbers:

| 0 −−−−− | 1 ·−−−− | 2 ··−−− | 3 ···−− | 4 ····− | 5 ····· |
| --- | --- | --- | --- | --- | --- |
| 6 −···· | 7 −−··· | 8 −−−·· | 9 −−−−· |  |  |

Punctuation:

| . period ·−·−·− | , comma −−··−− | ? question ··−−·· | / slash −··−· | ( open −·−−· | ) close −·−−·− |
| --- | --- | --- | --- | --- | --- |
| : colon −−−··· | ; semi −·−·−· | - hyphen −····− | ' apos ·−−−−· | " quote ·−··−· | @ at ·−−·−· |
| & amp ·−··· | ! excl −·−·−− | _ under ··−−·− | $ dollar ···−··− |  |  |

Prosigns:

| + AR ·−·−· end msg | = BT −···− break | ~ SK ···−·− end QSO | & AS ·−··· wait |  |  |
| --- | --- | --- | --- | --- | --- |

K0WLY Two-Way CW Keyer — User Guide v1.6.0 — 73 de K0WLY